module FOURNAND(

input a,b,c,d,

output e,f,g

);

assign e = ~(a&b);

assign f = ~(e&c);

assign g = ~(f&d);

endmodule

#############################

set_property -dict {PACKAGE_PIN J4 IOSTANDARD LVCMOS18} [ get_ports a ]

set_property -dict {PACKAGE_PIN L3 IOSTANDARD LVCMOS18} [ get_ports b ]

set_property -dict {PACKAGE_PIN K3 IOSTANDARD LVCMOS18} [ get_ports c ]

set_property -dict {PACKAGE_PIN M2 IOSTANDARD LVCMOS18} [ get_ports d ]

set_property -dict {PACKAGE_PIN F15 IOSTANDARD LVCMOS18} [ get_ports e ]

set_property -dict {PACKAGE_PIN F13 IOSTANDARD LVCMOS18} [ get_ports f ]

set_property -dict {PACKAGE_PIN F14 IOSTANDARD LVCMOS18} [ get_ports g ]

#################################

module FOURNAND_tb;

reg aa, bb, cc, dd;

wire ee, ff, gg;

FOURNAND u_test(

.a(aa),

.b(bb),

.c(cc),

.d(dd),

.e(ee),

.f(ff),

.g(gg)

);

initial begin

aa = 1'b0;

bb = 1'b0;

cc = 1'b0;

dd = 1'b0;

end

always@(aa or bb or cc or dd) begin

aa <= #50 ~aa;

bb <= #100 ~bb;

cc <= #150 ~cc;

dd <= #200 ~dd;

end

initial begin

#1000

$finish;

end

endmodule

module FOUROR(

input a,b,c,d,

output e,f,g

);

assign e = a|b;

assign f = e|c;

assign g = f|d;

endmodule

set_property -dict {PACKAGE_PIN J4 IOSTANDARD LVCMOS18} [ get_ports a ]

set_property -dict {PACKAGE_PIN L3 IOSTANDARD LVCMOS18} [ get_ports b ]

set_property -dict {PACKAGE_PIN K3 IOSTANDARD LVCMOS18} [ get_ports c ]

set_property -dict {PACKAGE_PIN M2 IOSTANDARD LVCMOS18} [ get_ports d ]

set_property -dict {PACKAGE_PIN F15 IOSTANDARD LVCMOS18} [ get_ports e ]

set_property -dict {PACKAGE_PIN F13 IOSTANDARD LVCMOS18} [ get_ports f ]

set_property -dict {PACKAGE_PIN F14 IOSTANDARD LVCMOS18} [ get_ports g ]

module FOURNOR_tb;

reg aa, bb, cc, dd;

wire ee, ff, gg;

FOUROR u_test(

.a(aa),

.b(bb),

.c(cc),

.d(dd),

.e(ee),

.f(ff),

.g(gg)

);

initial begin

aa = 1'b0;

bb = 1'b0;

cc = 1'b0;

dd = 1'b0;

end

always@(aa or bb or cc or dd) begin

aa <= #50 ~aa;

bb <= #100 ~bb;

cc <= #150 ~cc;

dd <= #200 ~dd;

end

initial begin

#1000

$finish;

end

endmodule

module FOURNOR(

input a,b,c,d,

output e,f,g

);

assign e = ~(a|b);

assign f = ~(e|c);

assign g = ~(f|d);

endmodule

############################